RED TORCH SECURITY SOLUTIONS LLC

RTSS SENTINEL PLATFORM

Current Technical Architecture

As-Built System Documentation

September 2026

CONFIDENTIAL — BUSINESS PROPRIETARY

# 1. System Overview

The RTSS Sentinel platform is a locally deployed, AI-driven physical security intelligence system running on a single NVIDIA Jetson Orin Nano Super at the client's facility. All processing — video ingestion, AI inference, person and vehicle tracking, identity management, zone-based alerting, and event storage — executes entirely on-premises. No data leaves the property. No cloud services are required for any function.

The system connects to seven Hikvision IP cameras via an NVR at 192.168.4.88 using RTSP over TCP. The management interface is a locally-hosted React web application served by FastAPI on port 8080, accessible from any device on the client's network.

# 2. Hardware Platform

## 2.1 Compute Module

| Specification | Value |
| --- | --- |
| Compute Module | NVIDIA Jetson Orin Nano Super |
| AI Performance | 67 TOPS (INT8) |
| System Memory | 8 GB LPDDR5 |
| GPU Cores | 1024 CUDA cores |
| CPU | 6-core Arm Cortex-A78AE |
| Power Envelope | 7–15W configurable TDP |
| JetPack Version | 6.2 (CUDA 12.6, TensorRT 10.3) |
| Primary Storage | NVMe SSD |
| Operating System | Ubuntu 22.04 (L4T) |

## 2.2 Camera Infrastructure

Seven Hikvision IP cameras connected through an NVR. RTSP streams are accessed on port 9090 with TCP transport forced for reliability. Sub-streams at 720p/1080p feed the detection pipeline at 10–15 FPS per camera. The system uses ONVIF and RTSP protocols and is designed to work with any ONVIF-conformant camera.

| Camera ID | Name | Stream Path |
| --- | --- | --- |
| cam-01 | Camera 1 | Streaming/Channels/102 |
| cam-02 | Camera 2 | Streaming/Channels/202 |
| cam-03 | Camera 3 | Streaming/Channels/302 |
| cam-05 | Camera 5 | Streaming/Channels/502 |
| cam-06 | Camera 6 | Streaming/Channels/602 |
| cam-07 | Camera 7 | Streaming/Channels/702 |
| cam-08 | Camera 8 | Streaming/Channels/802 |

# 3. Software Architecture

The software stack is organized into five layers. Each layer handles a distinct function and communicates with adjacent layers through defined internal interfaces.

## 3.1 Layer 1 — Hardware Abstraction Layer (HAL)

Purpose: Decouple all AI inference from the specific hardware. Every layer above calls the HAL interface and never directly touches TensorRT or CUDA.

Interface: Abstract base class InferenceBackend with methods: load_model(), detect(), detect_and_track(), get_status(), unload().

Current Backend: TensorRTBackend — compiles ONNX models to TensorRT FP16 engines on the Jetson GPU. Achieves 35+ FPS inference throughput.

Mock Backend: MockBackend returns synthetic detections for development and testing without GPU hardware.

Future Backend: THRMLBackend stub defined for eventual Extropic TSU thermodynamic computing integration.

## 3.2 Layer 2 — Video Ingestion Pipeline

Purpose: Discover cameras, connect RTSP streams, decode video frames, and route them to the detection engine.

CameraManager handles camera lifecycle — discovery, connection, reconnection with exponential backoff, and health monitoring. CameraStream runs a dedicated capture thread per camera with GStreamer hardware-accelerated decode (NVDEC) and fallback to FFmpeg. TCP transport is forced via OPENCV_FFMPEG_CAPTURE_OPTIONS for Hikvision compatibility. Camera configurations persist to JSON at ~/rtss/configs/cameras.json and auto-load on startup.

## 3.3 Layer 3 — Detection, Tracking, and Re-Identification Engine

### 3.3.1 Object Detection

Primary model: YOLO11s (small variant, ~9M parameters) compiled to a TensorRT FP16 engine. Processes each frame in a single forward pass, outputting bounding boxes with class labels and confidence scores. Detected classes include person, car, truck, bus, motorcycle, bicycle, dog, cat, bird, and other COCO classes.

Class normalization: car, truck, and bus are grouped as 'Vehicle' for display purposes. Dog, cat, bird, horse, cow, sheep, and bear are grouped as 'Animal'. The raw class is preserved in the database for filtering.

### 3.3.2 False Positive Filtering

Class-specific size and aspect ratio filters suppress false positives:

- Person: minimum 40×80 pixels, aspect ratio > 1.2 (taller than wide), minimum area 4,000 px²

- Vehicle: minimum area 8,000 px², minimum width 60px, minimum height 40px

- Animal: minimum area 1,500 px²

Bounding boxes only render on camera overlays for detections above 70% confidence.

### 3.3.3 Multi-Object Tracking

ByteTrack maintains persistent track IDs within each camera's field of view using Kalman filtering for motion prediction and the Hungarian algorithm for detection-to-track association. Track IDs persist through brief occlusions.

## 3.3.4 Person Re-Identification

OSNet x0.25 (Omni-Scale Network) generates 512-dimensional appearance embeddings from detected person crops. This is appearance-based — it encodes clothing color, texture, body proportions, and carried objects without computing facial geometry.

Two-tier gallery matching:

- Tier 1 — Known People: centroid-based matching at threshold 0.68. Each known person's centroid embedding is the average of all their reference embeddings.

- Tier 2 — Transient Sessions: individual embedding matching at threshold 0.65. New detections compared against recent anonymous session embeddings.

Batch matching ensures each identity token is assigned at most once per frame, preventing duplicate assignments.

## 3.3.5 Vehicle Re-Identification

ResNet18 backbone extracts 512-dimensional appearance embeddings from vehicle crops. Unlike color-based identification, this works in both day and night/IR camera modes. Cosine similarity matching at threshold 0.70 tracks vehicles across frames and cameras.

The VehicleTracker maintains two categories: transient vehicles (evicted after 10 minutes TTL) and permanent vehicles (labeled by the operator, never evicted). Known vehicles load from SQLite on startup with averaged centroid embeddings.

## 3.3.6 Session-Based Tracking

Person detections are organized into sessions rather than individual events. A session opens on first appearance and closes after 5 minutes with no detections (configurable via RTSS_REID_GALLERY_TTL_MINUTES). Sessions track:

- Person identity (known name or anonymous token)

- Category (household, known, unknown)

- Property status (unknown, on_property, inside, left) — determined by entry/exit zones

- Camera transitions (which cameras the person moved between)

- Position trail (foot-point coordinates per frame)

- Thumbnail (quality-based upgrades using Haar cascade face detection: 0=none, 1=body, 2=small face, 3=large face)

Closed sessions are persisted to the session_history SQLite table with pickled embeddings, enabling labeling and merging from Recent Visits even after the gallery has evicted the session.

## 3.4 Layer 3.5 — Identity Manager

The identity system manages two parallel enrollment galleries: people and vehicles.

### 3.4.1 People

Enrollment methods: manual operator tagging from the Who's Here page. An operator selects an active or recently closed session and assigns a name and category (household, known, unknown). The session's appearance embeddings become reference embeddings for that identity.

Embeddings auto-accumulate: the detection loop saves new embeddings with per-detection crop thumbnails for known people on every detection, up to 50 per person. Retention prunes above 50, keeping the 30 most recent. The centroid updates continuously.

The embedding gallery UI shows each embedding with its crop thumbnail, camera, timestamp, and cosine similarity to the centroid. Outliers (similarity < 0.70) are flagged. Individual embeddings can be removed to clean the gallery.

### 3.4.2 Vehicles

Vehicle enrollment follows the same pattern: Label from the Active Vehicles tab, assigning a name (e.g., 'My Car') and category (household, known). The vehicle's ResNet18 embedding is saved as a reference. Merge combines two vehicle IDs. Dismiss removes a transient vehicle from tracking.

Known vehicles are stored in the known_vehicles SQLite table with embeddings in vehicle_embeddings. They load into the tracker on startup as permanent entries with averaged centroids.

## 3.5 Layer 4 — Intelligence Engine

### 3.5.1 Zone-Based Detection

Operators define detection zones as polygons overlaid on camera views. Zone types control behavior:

| Zone Type | Severity | Behavior |
| --- | --- | --- |
| Restricted | Critical | Triggers critical alert on any detection |
| Perimeter | Warning | Triggers warning alert |
| Entry | Info | Marks session property_status as 'on_property' |
| Exit | Info | Marks session property_status as 'left' |
| Monitored | Info | Logs for analytics, info-level alert |
| Ignore | None | Suppresses all alerts and tracking in this area |

Each zone has configurable alert classes (person, vehicle, animal) and optional schedules with overnight range support (e.g., 22:00 to 06:00). Point-in-polygon testing uses the ray casting algorithm on the detection foot-point (bottom center of bounding box). Unknown persons in restricted or perimeter zones are escalated to critical severity.

### 3.5.2 Event Storage

All detection events are stored in a local SQLite database (rtss_events.db). Events include: event_id, camera_id, timestamp, class_name, confidence, bounding box, zone info, severity, session_token, vehicle_id, vehicle_label, and a JPEG crop thumbnail. Person events are throttled to one per 5 seconds per camera per class to prevent database bloat from 15 FPS detections.

### 3.5.3 Vehicle and Animal Presence Tracking

Vehicles use embedding-based re-identification — the same vehicle produces one 'arrived' alert and one 'departed' alert regardless of how long it stays. Animals use a simpler cooldown-based approach: same class on same camera suppressed for 120 seconds.

### 3.5.4 Data Retention

Automated cleanup runs on startup and every 6 hours:

- Events: deleted after 30 days (configurable)

- Embeddings: per-person capped at 50, pruned above that keeping the 30 most recent

- Baselines: deleted after 90 days

- Storage watchdog: if disk exceeds 90%, halves retention window and strips old thumbnails

### 3.5.5 Activity Feed

The /api/activity endpoint aggregates session events, vehicle arrivals/departures, and zone alerts from three sources: in-memory active sessions, the session_history database table (for closed sessions surviving restarts), and the events table. Results are deduplicated and enriched with current labels from known_vehicles and known_people tables.

## 3.6 Layer 5 — Management Interface

The client-facing application is a React 18 single-page application built with Vite, styled with Tailwind CSS v4, and served as static files by the FastAPI backend. No internet connection is required.

### 3.6.1 Dashboard

Live camera grid with adaptive refresh: 500ms idle, 67ms (15 FPS) when WebSocket alerts reference that camera. Active window of 5 seconds. System health telemetry polling at 250ms (4 Hz). Activity feed with Live and 24h History tabs, filterable by People, Vehicles, and Zones.

### 3.6.2 Who's Here (People)

Three tabs: Active Sessions (currently detected people), Recent Visits (closed sessions from the last 24 hours), and Known People. Each session card shows face thumbnail, name, category badge, property status, camera, and duration. Actions: Label, Merge (into an existing known person), Dismiss. Known person cards link to a detail page with embedding gallery.

### 3.6.3 Vehicles

Two tabs: Active Vehicles (currently tracked) and Known Vehicles. Each vehicle card shows a crop thumbnail, vehicle ID, camera, and time. Actions: Label, Merge, Dismiss. Known vehicle cards link to a detail page with embedding gallery and similarity scores.

### 3.6.4 Cameras

Camera management with snapshot preview, connection status, FPS, and drop count. Zone drawing tool: click to place polygon points on a camera snapshot, configure zone type, alert classes, and schedule. Existing zones displayed with color-coded overlays.

### 3.6.5 Events

Filterable event history with stat bar (total, critical, warning, people, vehicles, animals, zone hits, sessions). Filters by camera, severity, class (People, Vehicles, Animals), and time range (1h to 7 days). Each event row shows a crop thumbnail, severity badge, class, camera, confidence, zone name, and clickable session token.

### 3.6.6 Property Map

Upload a property image (satellite screenshot or floor plan). Place cameras on the map with configurable direction (0–359°), field of view (20–180°), and reach. Camera FOV cones rendered as SVG overlays. Live view shows person dots with face thumbnails at camera positions, movement trails between cameras, compass rose (counter-rotates with map rotation), and calibrated scale bar. Map rotation and scale persist to SQLite.

### 3.6.7 Settings

System info (hardware, JetPack, CUDA, TensorRT), storage metrics, detection configuration, data retention stats with manual cleanup trigger, camera list, and zone list.

### 3.6.8 Real-Time Communication

WebSocket endpoint at /ws/alerts pushes detection events, session lifecycle events (arrived, departed, camera change), zone violations, and vehicle/animal arrivals to all connected dashboard clients. Monotonic message IDs prevent duplication.

# 4. Backend API Structure

FastAPI (Python ASGI) backend with the following route modules:

| Route Prefix | Module | Purpose |
| --- | --- | --- |
| /api/cameras | cameras.py | Camera CRUD, snapshots with detection overlay |
| /api/events | events.py | Event history with pagination and filtering |
| /api/sessions | sessions.py | Active/closed sessions, labeling, merging, thumbnails, embedding galleries |
| /api/zones | zones.py | Zone CRUD, polygon drawing |
| /api/vehicles | vehicles.py | Vehicle tracking, labeling, merging, embedding galleries |
| /api/health | health.py | System telemetry (GPU, CPU, memory, disk, inference FPS) |
| /api/map | property_map.py | Property map image, camera positions, rotation, scale, live data |
| /api/activity | activity.py | Aggregated activity feed for dashboard |
| /api/retention | retention.py | Data retention stats and manual cleanup |
| /api/alerts | alerts.py | Alert management (stub) |
| /api/auth | auth.py | JWT authentication (implemented, bypassed in dev) |
| /ws/alerts | WebSocket | Real-time alert streaming |

# 5. Database Schema

All data is stored in a single SQLite database file (rtss_events.db). Tables:

| Table | Purpose | Key Fields |
| --- | --- | --- |
| events | All detection events | event_id, camera_id, timestamp, class_name, confidence, bbox, zone, severity, session_token, vehicle_id, thumbnail |
| known_people | Enrolled person identities | id, name, category, is_active, created_at |
| people_embeddings | Reference embeddings per person | id, person_id, embedding (512 floats), camera_id, thumbnail, captured_at |
| people_thumbnails | Best face thumbnail per person | person_id, thumbnail, quality, updated_at |
| session_history | Closed sessions (persists across restarts) | token, person_name, category, cameras, events (JSON), embeddings (pickled), thumbnail |
| known_vehicles | Labeled vehicle identities | vehicle_id, name, category, created_at |
| vehicle_embeddings | Reference embeddings per vehicle | id, vehicle_id, embedding (512 floats), thumbnail, captured_at |
| zones | Detection zone polygons | id, camera_id, name, zone_type, polygon (JSON), alert_classes, schedule |
| property_map | Map image and camera positions | image, camera_positions (JSON), rotation, scale_pixels, scale_meters |
| baselines | Pattern-of-life hourly baselines | camera_id, zone_id, hour_of_week, class_name, avg_count, std_dev |
| users | Dashboard user accounts | id, username, hashed_password, role |
| audit_log | User action log | timestamp, user_id, action, resource_type, justification |

# 6. Key Configuration

System configuration is managed through environment variables loaded from ~/rtss/.env via pydantic-settings:

| Parameter | Value | Description |
| --- | --- | --- |
| RTSS_TIER | residential | Product tier (residential/municipal/enterprise) |
| RTSS_DETECTION_CONFIDENCE | 0.5 | Minimum detection confidence from YOLO |
| RTSS_REID_THRESHOLD | 0.68 | Known person matching threshold |
| RTSS_REID_SESSION_THRESHOLD | 0.65 | Transient session matching threshold |
| RTSS_REID_GALLERY_TTL_MINUTES | 5 | Session timeout (departure after 5 min no detections) |
| RTSS_EVENT_RETENTION_DAYS | 30 | Days to keep detection events |
| RTSS_EMBEDDING_RETENTION_DAYS | 7 | Days before pruning excess embeddings |
| RTSS_MAX_STORAGE_PERCENT | 90.0 | Disk usage threshold for emergency cleanup |
| RTSS_MAX_CAMERAS | 8 | Maximum concurrent camera streams |
| RTSS_INFERENCE_FPS | 15 | Target inference rate per camera |
| RTSS_API_PORT | 8080 | Management interface port |

# 7. AI Model Pipeline

| Model | Architecture | Dimensions | Purpose | Latency |
| --- | --- | --- | --- | --- |
| YOLO11s | Single-stage detector | 640×640 input | Object detection (person, vehicle, animal) | ~28ms per frame |
| OSNet x0.25 | Lightweight CNN (torchreid) | 256×128 input → 512-d | Person re-identification | <5ms per crop |
| ResNet18 | CNN backbone (torchvision) | 224×224 input → 512-d | Vehicle re-identification | <5ms per crop |
| ByteTrack | Tracking algorithm | N/A | Single-camera multi-object tracking | <1ms per frame |
| Haar Cascade | OpenCV classifier | N/A | Face detection for thumbnail quality scoring | <2ms per crop |

All models are trained in PyTorch, exported to ONNX as the canonical format, and compiled to TensorRT FP16 engines on the Jetson. The ONNX files serve as the hardware-agnostic model archive for future hardware migrations.

# 8. Project Structure

rtss/

├── rtss/

│   ├── config.py                    — Pydantic settings with env loading

│   ├── main.py                      — Entry point (uvicorn launcher)

│   ├── hal/                          — Layer 1: Hardware Abstraction

│   │   ├── base.py                  — Abstract InferenceBackend interface

│   │   ├── tensorrt_backend.py       — TensorRT GPU inference

│   │   ├── mock_backend.py           — Mock for testing

│   │   └── thrml_backend.py          — Future TSU stub

│   ├── ingestion/                    — Layer 2: Video Pipeline

│   │   ├── camera_manager.py         — Camera lifecycle and config

│   │   ├── camera_stream.py          — RTSP capture with reconnection

│   │   └── camera_store.py           — JSON persistence for cameras

│   ├── detection/                    — Layer 3: Detection Engine

│   │   ├── zone_checker.py           — Point-in-polygon zone logic

│   │   ├── vehicle_id.py             — Vehicle embedder + tracker

│   │   └── models/                   — Model weights (.engine, .onnx)

│   ├── identity/                     — Layer 3.5: Identity Manager

│   │   ├── gallery.py                — Two-tier embedding gallery

│   │   ├── session_manager.py        — Session lifecycle tracking

│   │   ├── people_store.py           — SQLite persistence for identities

│   │   ├── embedding.py              — OSNet embedding extraction

│   │   └── property_map.py           — Map image + camera positions

│   ├── intelligence/                 — Layer 4: Intelligence Engine

│   │   ├── event_store.py            — SQLite event database

│   │   ├── pattern_of_life.py        — Baseline learning

│   │   └── retention.py              — Data retention cleanup

│   └── api/                          — Layer 5: Management Interface

│       ├── server.py                 — FastAPI app with SPA middleware

│       ├── state.py                  — Detection loop, app state singleton

│       └── routes/                   — API route modules

├── frontend/                         — React 18 + Vite + Tailwind v4

│   └── src/                          — Components, pages, hooks

├── configs/                          — cameras.json, default.env

├── scripts/                          — deploy.sh, build_engines.sh

└── systemd/                          — rtss-sentinel.service

# 9. Deployment

The system deploys as a systemd service (rtss-sentinel.service) with Restart=always and RestartSec=5 for automatic recovery. The deploy script (scripts/deploy.sh) builds the React frontend with Vite and starts the uvicorn server. The --run-only flag skips the frontend rebuild for faster restarts.

Detached operation: nohup ./scripts/deploy.sh --run-only > /tmp/rtss.log 2>&1 &

Logs are available at /tmp/rtss.log and via journalctl when running as a systemd service.

# 10. Measured Performance

| Metric | Value |
| --- | --- |
| YOLO11s inference | 35+ FPS (28ms/frame) at FP16 |
| OSNet embedding extraction | <5ms per person crop |
| ResNet18 vehicle embedding | <5ms per vehicle crop |
| Camera streams | 7 concurrent at 10-15 FPS each |
| GPU temperature | 46–52°C under sustained load |
| System power draw | 10–15W total |
| Memory usage | ~2.9 GB of 7.6 GB available |
| Disk usage | ~44 GB of 113 GB |
| Uptime | Sustained 3+ days continuous operation confirmed |

Red Torch Security Solutions LLC — Confidential